|  |  |  | Purpose of the Technical Specification / Scope of the Technical Specification Purchase Technical Specification Technical specification for purchase |  |  |
| --- | --- | --- | --- | --- | --- |
| Document number / Document no. TS/SWNFCH/2025 |  |  |  | Number of pages No. of pages | 24 |
| Project name Project name |  | Software for calculations of neutron-physical characteristics of reactor cores and nuclear fuels |  | Security Index Classification level |  |
| Rev no. | Description of Revisions to the technical specification |  |  |  |  |
| 00 |  |  |  |  |  |

Approval protocol:

|  | Job Position | Name | Date | Signature |
| --- | --- | --- | --- | --- |
| Prepared by | Team Leader for Nuclear Fuel and Reactor Cores | Róbert Kelemen |  |  |
| Co-operation | Head of the EBO Reactor Physics Group | Ľuboslav Paulík |  |  |
|  | Head of the EMO Reactor Physics Group | Silvia Belušáková |  |  |
|  | Corporate and Cyber Security Manager | Miloš Bednár |  |  |
|  | Head of IT production support systems | Peter Chvostál |  |  |
|  | IT Project manager | David Gieci |  |  |
| Submitted by | Head of Nuclear Fuel Projects and Assessment | Peter Páleník |  |  |
| Approved by | Fuel Cycle and Radwaste Manager | Miloš Lašček |  |  |
| Verified by | Deputy Director of the NPP Division | Patrik Kašo |  |  |

The requirements set out in this specification and in the attached documents are to be regarded as minimum requirements which do not limit the subject matter of the performance. The Contractor acknowledges that any materials, systems, and activities, even if not listed in this Technical Specification, but necessary for proper operation, shall be deemed to be included in the Scope of Work to make it fully functional, safe, and reliable for long-term operation.

Table of Contents

1.	Definitions and abbreviations	3

1.1	Definitions of terms	3

1.2	Abbreviations	3

2.	Baseline situation	4

2.1	Aim and purpose of the subject-matter	4

2.2	Description of the current situation and classification	4

2.3	Place of delivery	4

2.4	Characteristics of the environment	4

2.5	Supporting documents of SE	5

3.	Scope of delivery and options	6

3.1	Scope of delivery	6

3.2	Options	6

4.	Functional, safety and technical requirements	7

4.1	Basic criteria	7

4.1.1	SW solution qualification for VVER-440 reactors and qualification and experience of personnel	7

4.1.2	Comparison of calculation results with operating parameters and another code	8

4.2	Functional requirements	11

4.2.1	General requirements	11

4.2.2	Detailed features	11

4.3	Legislative and other requirements	14

4.4	Method of delivery	16

4.4.1	Approach to project implementation	16

4.4.2	Schedule	17

4.4.3	Project outputs	18

4.5	Operational services	21

5.	Limits of performance, exclusion from performance and counter-performance	23

5.1	Boundaries of performance	23

5.2	Counterclaims	23

5.2.1	Documentation to be provided by SE	23

6.	Annexes to the technical specification	24

# Definitions and abbreviations

## Definitions of terms

Supplier / vendor / contractor / applicant / tenderer

It is a natural or legal person with the required professional competence and a valid authorization for the performance of the activity, who, after the conclusion of the contract, performs the subject of performance for which the technical specification is being created.

Purchaser / customer

Slovenské elektrárne, a.s., Pribinova 40, 811 09 Bratislava, Slovak Republic Reg.No.: 35 829 052 Commercial Register of the Municipal Court Bratislava III, Section: Sa, No.: 2904/B.

## Abbreviations

AER		Atomic Energy Research

AO1		Fast automatic reactor shutdown (free fall of control rods) / SCRAM

DNBR		Departure from Nucleate Boiling Ratio

DR		Disaster recovery

EBO		Jaslovské Bohunice Nuclear Power Plant

EMO		Mochovce Nuclear Power Plant

ES		Energetical start-up

FS		Physical start-up

GUI		Graphical User Interface

HA		High Availability

ENDF		Evaluated Nuclear Data File

HW		Hardware

IAEA		International Atomic Energy Agency

ICT		Information and Communication Technologies

KASKAD	KASKAD software package for neutron-physics calculations

NFCH		neutron-physical characteristics

NR SR		National Council of the Slovak Republic

NPP		Nuclear power plant

PWR		Pressurized Water Reactor

SE		Slovenské elektrárne, a.s.

SCRAM	Automatic (emergency) reactor shutdown by free fall of control rods

SW		Software

VVER		Water-Water Power Reactor

TVEL		TVEL Fuel Company of Rosatom

ÚJD SR	Nuclear Regulatory Authority of the Slovak Republic

# Baseline situation

## Aim and purpose of the subject-matter

The objective of the subject of performance is to procure software for complex calculations and assessment of neutron-physical characteristics of reactor core loads for nuclear fuels from various suppliers and support.

The purpose of the subject is to create models and libraries for currently used nuclear fuels and for nuclear fuels from alternative nuclear fuel suppliers (Westinghouse, Framatome, etc.), to design reactor core loadings and to calculate neutron-physical characteristics.

The software must be specifically tailored for VVER-440 reactors to allow calculations for EBO and EMO reactors.

The software can be composed of several modules that communicate with each other or the outputs from one module are converted into inputs for another module without any additional intervention of the software user.

The software as a whole must provide the capability for stand-alone nuclear fuel modelling by users of the software for VVER-440 reactors, the creation of libraries for all necessary fuels, the design of reactor core loadings and neutron-physical parameters calculations.

The software vendor must be independent of the nuclear fuel vendors so that situations do not arise where nuclear fuel data from different nuclear fuel vendors cannot be shared with the software vendor.

## Description of the current situation and classification

Currently, the KASKAD software package is used for designing the reactor core loads and NFCH calculations. This software does not allow the user to create libraries for different nuclear fuels as it does not include a dedicated module. It can only perform calculations for nuclear fuels for which libraries have been supplied by the software vendor and which are used by the KASKAD software package in the NFCH calculations. For this reason, NFCH can only be calculated with nuclear fuels from TVEL.

In view of the need to perform NFCH calculations also for nuclear fuels from alternative nuclear fuel suppliers by the internal staff of the SE, it is necessary to procure new software that will fully replace the currently used KASKAD software package. At the same time, it is necessary that the new software also allows the creation of models and libraries for different types of nuclear fuels from different nuclear fuel suppliers, so that the whole process of NFCH calculation is fully within the competence of the SE.

## Place of delivery

Place of delivery is Slovenské elektrárne, a.s., Pribinova 40, 82109 Bratislava.

## Characteristics of the environment

SE is an operator of critical essential services pursuant to Act No. 69/2018 Coll. on Cyber Security and on Amendments and Additions to Certain Acts, as amended (hereinafter referred to as "Act No. 69/2018 Coll."), and must comply with the requirements of cyber security legislation. The general requirements for the area of cyber security are laid down by Act No. 69/2018 Coll. and Decree of the National Security Authority No. 227/2025 Coll. laying down the security measures (hereinafter referred to as "NSA Decree No. 227/2025 Coll.").

The subject of performance must be implemented regarding ensuring compatibility with existing networks and information systems and maintaining the level of cybersecurity of the affected networks and information systems of the SE by appropriately applying and complying with the requirements set out in the following annexes:

- ICT architectural and technological standards (Annex 1),

- Annex No. 1 to Decree No. 227/2025 Coll. (Annex 2).

## Supporting documents of SE

- ICT architectural and technological standards (Annex 1),

- Annex No. 1 to Decree No. 227/2025 Coll. (Annex 2),

- Core design project (Annex 3),

- Detailed neutron-physical characteristics of the reactor core (Annex 4),

- Neutron-physical characteristics of the reactor core for physical and energetical start-up tests (Annex 5),

- Computational code verification report (Annex 6).

The tenderer will be provided with this Technical Specification including all annexes.

# Scope of delivery and options

## Scope of delivery

The scope of delivery is:

- Delivery and installation of software by the supplier and support for local infrastructure setup.

- Software testing by the vendor after the software is installed, delivery of test cases for software testing by software users and support in launching and evaluating test cases, if required by the customer.

- Installation and administration documentation delivery.

- User documentation delivery.

- The report stating of the computational uncertainties of delivered system.

- Training of end users and their certification by the supplier.

- Graphical User Interface (GUI) for creating a core shuffling.

- Tool for generating the NFCH, Core Design Project and NFCH for physical and energetical start-up test reports.

- Assistance provided by the supplier to end users during implementation and testing of calculations for individual SE nuclear units

- Final Validation and Acceptance report

- Production License key (if needed)

- Technical support, software maintenance, resolving inconsistencies in calculations, software updates and upgrades.

The scope of supply shall be complete, with all equipment and accessories necessary for safe and reliable operation and shall include, but not be limited to, the goods, works and services specified in the technical specification.

The software shall meet the information systems and network requirements specified in this Technical Specification as well as the cyber security requirements. It must be installed on SE-owned HW assets.

## Options

They are not currently foreseen.

# Functional, safety and technical requirements

## Basic criteria

### SW solution qualification for VVER-440 reactors and qualification and experience of personnel

The Applicant must submit or declare:

###### Qualification of delivered SW

(a) Qualification for VVER-440

- The supplier must provide official documentation or a report confirming the qualification of the software solution for use of the software for VVER-440 reactors (qualification report or similar). This qualification must be issued or approved by a relevant local authority.

- If such documentation for VVER-440 reactors is not currently available, the supplier may provide proof of qualification for other types of VVER reactors and/or PWR reactors as an acceptable alternative (qualification report of the software solution or similar, issued or approved by a relevant local authority) as specified in tender documentation conditions of participation in public procurement, and must subsequently fulfill the requirement in accordance with point (b).

  - (b) Obtaining qualification for VVER-440

If qualification for VVER-440 reactors is currently not available and supplier provided documentation as specified in point (a), the second bullet point, the supplier is required to:

- Undertake all necessary actions to obtain qualification for VVER-440 from the relevant authority,

- Or provide support to Slovenské elektrárne (SE) in obtaining approval from the local authority in the Slovak Republic for the use of the software for VVER-440 reactors.

(c) Verification and Validation

If qualification for VVER-440 is not available, or if the qualification does not meet all the requirements defined for verification and validation as specified in Chapter 4.1.2, the supplier must conduct a verification and validation of the software. The details of this process are specified in chapter 4.1.2 of the technical specification.

Final Verification and Validation must be performed for the EBO and EMO units as an integral part of the implementation process of the delivered software solution specifically tailored for the EBO and EMO units. See milestone M3, output D7.

For the tool specified in Requirement 2.2.34, the Customer may grant an exception in justified cases, particularly with respect to the required SE-specific customizations.

###### Qualifications and experience of the personnel

Minimum required level of the standard:

The Tenderer must demonstrate their professional competence to provide the required performance by confirming that they have at their disposal the individuals designated for contract performance or managerial employees (specified experts below) who meet the established requirements in the appropriate number. The Tenderer shall submit a list of individuals proving their professional competence in the form of a completed "List of Professionals" form, which constitutes Annex No. 10 of Tender Documents.

The list must also include information about the type of contractual relationship these experts have with the Tenderer (employee, subcontractor's employee, sole trader).

The Tenderer must prove their professional capacity:

a) by presenting a professional CV’s for each project member containing at least the following:

- first name and surname.

- completed education;

- current job position;

- overview of professional practice relating to the performance of the tender subject matter;

- signature in own hand of the expert.

b) in the case that the Candidate is not the current employer of the relevant expert, by presenting a written contract entered into by and between the Candidate and the expert (pursuant to the identification below) concerning the use of professional capacity of the expert in performing the contract with the Candidate, provided that the Candidate succeeds in this public procurement.

c) Define role for each member in the project

The Tenderer must provide at least:

Expert no. 1)

Expert in the field of reactor engineering/physics

- minimum 5 years of professional experience in an area of equal or similar nature and complexity as that of the tender subject matter.

Areas considered equal or similar nature to the subject matter of project include:

  - development or implementation or maintenance and support of a reactor core analysis SW from the point of reactor physics,

  - and performance of neutronics calculations.

- Can be proved by:

  - CV - including information on education, professional experience, previous projects

  - Diploma

  - Certificates (if applicable)

Expert no. 2)

Expert in the field of software engineering

- minimum 5 years of professional experience in an area of equal or similar nature and complexity as that of the tender subject matter.

Areas considered equal or similar nature to the subject matter of the project include for reactor core analysis software design or development or enhancement or integration or upgrades and maintenance or support.

- Can be proved by:

  - CV - including information on education, professional experience, and previous projects

  - Diploma

  - Certificates (if applicable)

Expert no. 3)

Person responsible for training

- minimum 5 years of professional experience in an area of equal or similar nature and complexity as that of the tender subject matter.

Areas considered equal or similar nature to the subject matter of the project include providing trainings in reactor core analysis SW calculations and neutronics calculation.

- Can be proved by:

  - CV - including information on education, professional experience, and previous projects

  - Diploma

  - Certificates (if applicable)

Additional information:

  - One individual may perform the role of a maximum of two Experts (applies to Expert 1 and Expert 3 only). However, the Tenderer is required to nominate at least two different persons to demonstrate compliance with the requirements for all designated Experts.

  - Persons on whose capacity the Candidate relies on to prove the satisfaction of this condition must be involved in the performance of the contract. Such workers shall also be indicated in the contract.

Persons on whose capacity the Candidate relies on to prove the satisfaction of this condition must be involved in the performance of the contract. Such workers shall also be indicated in the contract.

### Comparison of calculation results with operating parameters and another code

Verification of the calculations of the offered software solution in the software development phase is usually carried out by the software supplier and confirms the correctness of the models and methodologies used. The calculations are compared with

- real operating data of the reactor unit,

- results from other verified codes,

- results from precision codes (benchmarks).

###### Comparison of calculation results

The supplier must declare the correctness of the modelling of the parameters by the offered software solution, comparing the selected parameters chosen so that their verification covers the verification of the other characteristics due to their interconnectedness. The accuracy and correctness of the calculations must be declared on the VVER-440 unit, where the results of the calculations by the offered software are compared with the results of the calculations by other verified software and with the real parameters of the unit. The comparison must be made on the VVER-440 reactor unit cycles.

The Bidder shall demonstrate performance by submitting a technical report at least to the extent specified in Requirement 1.2.1 - Requirement 1.2.5, taking into account that verification against operational data is required. However, verification against another verified code is not mandatory. Verification against a precision code is required. If comparison with operational data for a limited set of parameters is not feasible (e.g., due to the unavailability of operational data), it is acceptable to compare the results with those of another licensed computational code.

###### Comparison of calculation results with operational values

To assess the consistency of the calculation of the offered SW with the measured operating values of the reactor unit, the power peaking factors and selected characteristics of the reactor core are compared for which there are experimental values obtained from the results of the physical start-up tests.

In order to demonstrate the accuracy of the calculation of the offered SW, the following parameters or characteristics must be verified:

- Critical boric acid concentrations during physical start-up tests,

- The trend of the critical acid concentration at nominal power during the cycle,

- Reactor SCRAM worth and reactor SCRAM worth with one stuck rod during physical start-up tests,

- Temperature coefficient of reactivity, boric acid reactivity coefficient and efficiency of the working group of control rods during physical start-up tests,

- Power peaking factors in the reactor core: for fuel assemblies kq, fuel rods kr, nodes kv, fuel pellets k0.

The above parameters are verified by experimental values for at least three cycles. Criteria shall be established to assess the consistency of the parameters which respect the uncertainties of calculation and measurement.

Please take into account that verification against operational data is required. However, verification against another verified code is not mandatory. Verification against a precision code is required. If comparison with operational data for a limited set of parameters is not feasible (e.g., due to the unavailability of operational data), it is acceptable to compare the results with those of another licensed computational code.

###### Comparison of calculation results with another code

The range of parameters for verifying the correctness of the calculation of the offered SW with the calculation data of other verified SW must be chosen to cover all the most important characteristics of the reactor core, with emphasis on safety parameters. On this basis, it is necessary to confirm the correctness of the calculation for at least the following parameters:

- Power peaking factors in the reactor core: for fuel assemblies kq, fuel rods kr, nodes kv, fuel pellets k0,

- Reactivity coefficients and point kinetics parameters, critical parameters,

- Reactor SCRAM worth for basic states,

- Reactor SCRAM worth with the most effective rod stuck,

- Control rod from group 6 (working group) ejection worth,

- Worth of control rods group 6 (working group),

- Subcriticality of the reactor after SCRAM with the most effective rod stuck and cooled down, calculation of the secondary criticality temperature, subcriticality of the reactor after SCRAM when one of the control rods groups is stuck along with the most effective rod stuck.

Calculation verification must be performed for at least three cycles.

Please take into account that verification against operational data is required. However, verification against another verified code is not mandatory. Verification against a precision code is required. If comparison with operational data for a limited set of parameters is not feasible (e.g., due to the unavailability of operational data), it is acceptable to compare the results with those of another licensed computational code.

###### Comparison of calculation results with precision codes

Comparison with "precision codes" is also necessary, e.g. some of the AER Benchmark for VVER-440 (i.e. at least micro power distribution is required).

Please take into account that verification against operational data is required. However, verification against another verified code is not mandatory. Verification against a precision code is required. If comparison with operational data for a limited set of parameters is not feasible (e.g., due to the unavailability of operational data), it is acceptable to compare the results with those of another licensed computational code.

###### Code uncertainties

The code uncertainties of the offered SW solution must be defined at least for following parameters:

Fuel temperature reactivity coefficient

Moderator temperature reactivity coefficient

Moderator density reactivity coefficient

Boron reactivity coefficient

Power reactivity coefficient

Effective fraction of delayed neutrons

Lifetime of prompt neutrons

Worth of the working group of control rods

Worth of emergency protection (excluding the most effective control rod)

Worth of one control rod in the working group

Reactor SCRAM worth

Critical concentration of boric acid

Duration of fuel cycle operation

Poisoning effect of Xe-135

Temperature of re-criticality

Power peaking factors Kq, Kv, Kk uncertainties are for the following categories:

Kq (pHA) – power peaking factor of the fuel assembly:

- Relative power of the fuel assembly with the highest power,

- Relative power of peripheral fuel assemblies with burnup greater than 30 MW days/kg in reduced leakage mode,

- Relative power for other fuel assemblies.

Kv – power peaking factor of the layer in the core within a single fuel assembly (volume power peaking factor of the core).

Kk – pin power distribution in the fuel assembly:

- Power peaking factor of the fuel rod,

- Power peaking factor of the gadolinium fuel rod,

- Power peaking factor of the fuel rod adjacent to the gadolinium fuel rod.

From these values, uncertainties for Kr (FdH) = Kq*Kk and K0 (Fq) = Kv*Kk are derived.

Decay heat and activity of fuel uncertainties (Requirement 2.2.26)

If uncertainties evaluation for VVER-440 reactors is not available, the selected supplier (i.e., the winner of the tender) shall provide calculated uncertainties in the documentation for the software solution (Requirement 4.3.4 User Documentation), and, if necessary, SE will provide operational data. The output is stated also in the table of the chapter 4.4.3, milestone M1, output ID D4.

## Functional requirements

### General requirements

###### Software functionalities in general 1

The software shall be designed to perform neutron-physics calculations of the VVER-440 reactor cores. It shall provide the functionality to develop and optimize the design of the core loading and the design of the subsequent nuclear fuel reloads and their comprehensive evaluation in terms of neutron-physical parameters. It shall provide the capability to create the necessary libraries for the nuclear fuels used in the calculations using nuclear data no older than the ENDF/B-VII.1 library. The system shall allow modelling of fuel of any geometry applicable to the VVER 440 and modelling of fuel assemblies for the calculation of reactivity under various storage conditions (this functionality should be included in the lattice code). Typically, this is a solution that involves multiple computational codes/modules interconnected at some level.

- Computational code for computing effective cross-section libraries and modeling fuel assemblies (lattice physics code).

- Computational code for neutron-physics calculations of the reactor core.

- Code for calculating isotopic composition and residual power.

- Optimization code for loading pattern design of the reactor core.

- Thermo-hydraulic model of the reactor core.

###### Software functionalities in general 2

The software must meet the scope of performance specified in this Technical Specification. The software as a whole shall allow the user to model the nuclear fuel for the VVER-440 reactors independently, to create libraries for all the necessary fuels from different fuel suppliers, to design the reactor core loadings and to calculate the neutron-physical parameters. The code uncertainties and process of verification and validation are described in the chapter 4.1.2 of this Technical Specification.

### Detailed features

The software must provide the following detailed functions:

| Requirement no. | Description of requirement |
| --- | --- |
|  | Creating and modifying the geometry and defining the material properties of the fuel assemblies components of VVER-440 nuclear reactors: fuel pellets, fuel column, fuel rod and its cladding, conduction in the fuel-cladding gap, fuel assemblies grids, central tubes, self-powered neutron detectors, top and bottom nozzles of fuel assemblies, fuel assemblies shroud, absorber parts, reflectors, water volumes, central openings in the fuel, imitators, etc. |
|  | Modeling of fuel rods, with the possibility of creating gadolinium rods, in-core instrument channels and water volumes. |
|  | Specification of uranium enrichment (mixtures of uranium) and gadolinium (burnable absorber) in fuel. |
|  | Creation of fuel rods in a triangular pitch into individual hexagonal shaped fuel assemblies used in VVER-440 reactors. |
|  | Creation of fuel assemblies with all components and specified materials into the reactor core model with a reflector. |
|  | The system should be able to model different flow rates through the fuel assemblies (inlet temperatures). |
|  | The code must allow the preparation of cross-sections libraries in a form that makes them available for universal use. It means that the outputs of the code (libraries) are in a form other than binary, described and with a known structure. |
|  | Modeling of fuel assemblies to calculate reactivity under various storage conditions within lattice code. For example, to model systems for fresh fuel assemblies in the fresh fuel storage, in a container, etc., as well as spent fuel in the spent fuel pool, and so on. |
|  | For the description of the position of the fuel assemblies in the reactor core, the cell annotation in 60° symmetry (1 to 59) / sixth number or, in justified cases, Cartesian coordinates must be used. For full core description of the positions of the fuel assemblies in the reactor core, 1 to 349 is required or, in justified cases, Cartesian coordinates must be used. |
|  | Calculation of VVER-440 fuel assemblies and fuel rods burnup. |
|  | Calculation of concentrations of individual isotopes as a function of burnup for at least following isotopes: U, Pu, Sm, Xe chains. |
|  | Calculation of 3D fuel burnup fields, power and neutron flux distributions, temperature and coolant density. |
|  | Simulation of the reactor core state during fuel burnup, xenon and samarium transients with consideration of realistic changes in operating parameters. |
|  | Calculation of power distribution limit parameters. |
|  | Calculation of absolute and relative power of fuel assemblies and fuel rods. |
|  | Calculation of the efficiency of control rods (reactor SCRAM worth). |
|  | Determination of the most efficient control rod (control rod with the highest worth) for a given control rods configuration in reactor core. |
|  | Calculation of reactivity coefficients and effects. |
|  | Calculation of point kinetics parameters. |
|  | Determination of the secondary criticality temperature and shutdown concentration of boric acid. |
|  | Calculation of DNBR, coolant outlet temperature from the fuel assembly and hot channel, fuel and fuel rod cladding temperature - the program shall include advanced thermohydraulic calculations. |
|  | Calculation of linear power of fuel rods and power ramps in local linear power in the fuel rod after fuel reload as a function of mean rod burnup depth/respectively burnup depth at a given fuel rod location, with a function to check the results of linear power and power ramps in linear power against a limit curve for these parameters. Further, evaluation of the linear power ramps for any two states. |
|  | Designing the first reactor core loading and further reloadings after the completion of each cycle, including the possibility of reloading nuclear fuel from the spent fuel pool. |
|  | Optimization of core loading pattern in terms of power peaking factors and minimization of the need of fresh nuclear fuel, with the possibility of disabling the movement of user-selected cells, with the possibility of disabling the placement of fuel assemblies with a residence year greater than a user-defined value, or with a burnup greater than a user-defined value in user-selected cells. The software solution must support this functionality in the following modes (at least ((a) and (b)) or ((a) and (c)) is required): - Manual Mode – all positions in the core can be defined manually by the user (manual fuel shuffling). - Semi-Automatic Mode – Simplified fuel assembly optimization that uses the principles of local optimization. The idea is to reduce the value of the objective function (in this case, the power distribution (kq and kr) over the entire cycle) by swapping several fuel assemblies within a symmetry sector (1/6, 1/12). Must allow possibility that user can lock specific fuel assembly positions in the core, and the software will automatically calculate and optimize the remaining positions based on power distribution. - Automatic Mode – Fully automatic optimization of core loading pattern. |
|  | Graphical User Interface (GUI) for creating a core shuffling. The environment shall be capable of creating a load in both 60° and 360° symmetry. The environment shall be capable of moving fuel assemblies with the mouse: single fuel assemblies, sixth symmetric fuel assemblies (all six symmetric fuel assemblies are moved at once) and twelfth symmetric fuel assemblies (all 12 symmetric fuel assemblies are moved at once). The system must allow a quick recalculation of the power distribution on the selected days (possibly in a simplified way). |
|  | Calculation of decay heat and fuel activity for reactor and spent fuel pool. |
|  | Generation of output text files (or similar format) with the results of the calculations. |
|  | Generation of calculation outputs in graphical form. |
|  | Generation of outputs to "open" database files and a GUI environment to work with these outputs. |
|  | Calculation of the influence coefficients on the measuring chambers e.g. for a SCRAM test with the most effective control rod stuck. |
|  | Ensuring the interaction of individual modules within the software including data exchange, error handling, and compatibility of data formats. |
|  | Storage of the necessary calculated data. |
|  | Input of reactor core operating data and simulation of calculations on the operating data. |
|  | A tool for generating the NFCH, Core Design Project and NFCH for physical and energetical start-up test reports (see Annexes 3-6 of this Technical Specification) delivered in a form that allows Customer's users to access the individual scripts, so that Customer's users have the ability to modify these scripts and add new ones as needed (Customer will grant an exception in justified cases). All reports must be generated in Word format (O365) with fully editable content and the ability to manage different layouts/templates. The language of the reports shall be in Slovak. Current Reports in Slovak to be provided to the winner of this public tender. The supplier must also deliver input files for the tool, along with calculation batches for the core simulation software and the lattice code. |
|  | The software must be delivered in such a way that calculations can be performed from multiple locations and by multiple users simultaneously (total expected number of users 15). |
|  | The software solution shall allow calculations and analyses at least to the extent necessary to enable the preparation of the documents listed in Annex 3-6 of this Technical Specification. |

## Legislative and other requirements

###### Compliance with international legislation

The following international legislative standards are related to the subject matter:

- IAEA Specific Safety Guide No. SSG-61 Format and Content of the Safety Analysis Report for Nuclear Power Plants

- IAEA Specific Safety Guide No. SSG-73 Core Management and Fuel Handling for Nuclear Power Plants

###### Compliance with national legislation

The following national legislative standards are related to the subject matter:

- Act of the National Council of the Slovak Republic No. 541/2004 Coll. on the Peaceful use of nuclear energy (Atomic Act) and on the amendments and supplements to some acts as amended

- Decree of the Nuclear Regulatory Authority of the Slovak Republic No. 430/2011 Coll. on nuclear safety requirements as amended by Decree of the Nuclear Regulatory Authority of the Slovak Republic No. 103/2016 Coll.

- Decree of the Slovak Nuclear Regulatory Authority No. 431/2011 Coll. on the quality management system as amended

- Security Guide BN 1/2019 - Quality assurance requirements for the safety analysis software (4th edition – revised and supplemented)

- Safety Guide BN 3/2020 - Requirements on design, realization and evaluation of results of reload startup physics tests (3rd edition – revised and supplemented)

- Safety Guide BN 5/2022 - Format and content of the safety analysis report (3rd edition – revised and supplemented)

- Safety Guide BNS III.4.3/2000 - Requirements on assessment of fuel loading for VVER 440 reactors

- Act of the National Council of the Slovak Republic No. 124/2006 Coll. on Occupational Health and Safety and on Amendments and Additions to Certain Acts.

- Act of the National Council of the Slovak Republic No. 69/2018 on cyber security and on amending and supplementing certain acts

- Decree of the National Security Office No 362/2018 Coll., establishing the content of security measures, the content and structure of security documentation and the scope of general security measures

- Slovak Government Regulation No. 396/2006 Coll. on minimum safety and health requirements for construction sites.

###### Solution architecture requirement

The software shall comply with the information systems and networking requirements specified in this Technical Specification:

Annex 1: ICT architectural and technological standards

In the event of a discrepancy, the Applicant shall be required to point out and explain the discrepancy. In justified cases, the Customer may grant an exception to such points.

###### Requirement for cybersecurity

The software is classified as supporting asset and must meet cyber security requirements defined in Annex 1 and Annex 2.

## Method of delivery

### Approach to project implementation

Starting point: we assume that the Applicant has application and software tools that:

  - meets most of the requirements listed in the specification.

  - requires a minimum level of additional implementation in order to meet the requirements set out herein.

  - it needs to be configured.

  - installation, configuration and user training are required for its use.

###### Requirement for license rights

Company Slovenské elektrárne, a.s. will gain permanent license to use the supplied software solution installed on-premises for an indefinite period.

In the event of a discrepancy, the Applicant shall be required to point out and explain the discrepancy. In justified cases, for third-party software only, the Customer may grant an exception to such points.

###### Requirement for ownership of source code of custom development

Slovenské elektrárne, a.s. needs to acquire ownership of part of the source code of custom software developed specifically for SE a.s. (report generating tool).

###### Project management

The Supplier will manage project according to the Prince2 or PMI project management methodology like:

  - Creation and maintain of project management documentation (WBS, project plan, partial plans, risk and issue register, Meeting of Minutes, tasks, etc.)

  - Meeting organization (kick off, regular status, subject metter meetings, steerings,…)

  - Regular project status Reporting

  - Test management (preparation, documentation, execution management and communication, progress tracking, reports, acceptance)

### Schedule

###### Contract milestones

The bidder shall add the contract milestone dates to the bid in a table, relative, in months from the signing of the purchase order/contract (example: 6.m means by the end of the 6th month from the signing of the contract):

| Milestone ID | Activity | Duration (in months) | Start at (month*) | End (month*) | Expected milestones by SE |
| --- | --- | --- | --- | --- | --- |
| M1 | Solution delivery*, installation, demonstration of functionality and correctness of calculations (Requirement 4.3.1 and Requirement 4.3.2) |  |  |  | 4. month |
| - M2 | Trainings (Requirement 4.3.5) |  |  |  | 12. month |
| - M3 | Development, Testing and Acceptance, Verification and Validation Report (Requirement 4.3.6) Methodological and application support and assistance to end users for solution start-up and calculations for individual SE nuclear reactors (Requirement 4.3.7) |  |  |  | 15. month |

* specify in months from the contract signature.

Contract milestones will be added to the contract between the Customer and the Supplier.

Invoicing milestones will be subject to the contract between the Customer and the Supplier.

The subject of billing is the accepted outputs in a given payment milestone.

###### Detailed schedule

The bidder shall submit a schedule with the bid that lists the activities in weekly granularity.

### Project outputs

The chapter lists and describes the outputs of the project.

The following outputs will be the subject of the project:

| Milestone ID | Output ID | Description, form | Method of acceptance |
| --- | --- | --- | --- |
| M1 | D1 | Delivery of software with installation support and demonstration of functionality and correctness of calculations (Requirement 4.3.1 and Requirement 4.3.2) | Tests by the Supplier Test results, with the maximum number of defects: Defect A - 0, Defect B - 0, Defect C - 10 |
| M1 | D2 | Installation and operational documentation (Requirement 4.3.3) | Confirmation on the acceptance report |
| M1 | D3 | User documentation (Requirement 4.3.4) | Confirmation on the acceptance report |
| M1 | D4 | A report stating the computational uncertainties of the system (Requirement 1.2.5) | Confirmation on the acceptance report |
| M2 | D5 | User trainings (Requirement 4.3.5) | Attendance list, qualification certificates of trained users |
| M3 | D6 | Development of GUI with manual, semi-automatic, automatic optimization for shuffling, Tool for generating reports, Testing and Acceptance. Permanent product license key (Requirement 4.3.6) | Tests by the User (UAT - user acceptance testing) Test results, with the maximum number of defects: Defect A - 0, Defect B - 0, Defect C - 10 |
| M3 | D7 | Verification and Validation Report for final solution for VVER-440 units (Requirement 4.3.6 ) | Confirmation on the acceptance report |
| M3 | D8 | Methodological and application assistance to end users during implementation of solution (Requirement 4.3.7) | Statement of Work approved by SE as the Customer |

###### Software delivery

- The delivered solution, after installation, verified by the Supplier when the Supplier demonstrates that the requirements specified in this Technical Specification are met. If the Requirement 2.2.24 , Requirement 2.2.25 and Requirement 2.2.34  are not included in the basic solution installation during milestone M1/D1, they must be delivered as part of milestone M3/D6.

- All files, scripts needed to install the solution.

- License keys for implementation and test phase (if needed)

- The Supplier shall provide a list of all third-party software components included in the delivered solution, including:

  - Name of the software

  - Manufacturer / owner of the software

  - Licensing terms (commercial, open-source, proprietary license)

  - Purpose of use within the solution

  - Information on compatibility and support

This list must be included in the installation and operational documentation (see Requirement 4.3.3 – Installation Documentation).

In case of any changes or additions to third-party software components during the lifecycle of the solution, the Supplier is obliged to update the list and inform the Customer accordingly.

###### Demonstration of SW functionality and correctness of calculations

During the commissioning phase of the software at the customer's site, it is necessary to verify and prove on at least one customer computer/laptop that the delivered and installed software is functional and at the same time it is necessary to confirm the correctness of the calculations after the software installation.

For this purpose, the tests need to be performed by the supplier after the installation of the software on the workstations, or test cases need to be delivered together with the software installation to verify the functionality and correctness of the software calculations by the user, and the supplier will provide support at the customer's request. The above shall be verified on at least one computer/laptop of the customer.

Testing of functionality and correctness of calculations includes:

- to run the software,

- verification of availability of all modules,

- performing basic functionality tests for each module (module loading, test data, verification by calculation) must be carried out for all delivered software modules.

For this purpose, the SW supplier shall issue a report demonstrating the functionality of the SW and the correctness of the calculations.

###### Installation and operational documentation

Installation and operational/administration documentation (in editable form), based on the SE as the Customer will be able to install and operate the solution.

###### User documentation

User documentation that includes:

- Description of each software module, GUI and Tools, its use, limitations, algorithms, methods and theoretical models.

- A user manual to the extent necessary to prepare the documents in Annexes 3-6, as well as examples of solved problems.

###### User training

The supplier must provide training for the customer's personnel. The date of the training will be agreed between the supplier and the customer in the contract:

- A minimum of 3 stages of training is expected - basic training followed by a short period of a few weeks to allow participants to repeat the calculations performed during the basic training, and then advanced trainings in a further 2 stages in such depth that participants will be able to perform calculations independently with each software module. At the end of the advanced training, a qualification certificate will be issued to each participant.

- The trainings should be carried out at the supplier's, customer's or other location as agreed between the supplier and customer.

- Trainings must include training materials, which will be provided at least 4 weeks prior to the start of each stage of training.

- The Supplier will provide IT administration training for technical personnel and consultants.

- The next stages of training (additional training) can be arranged based on the customer's request.

###### Development of GUI with manual, semi-automatic, automatic optimization for shuffling, Tool for generating reports, Testing, and Acceptance, final Verification and Validation report delivery

The supplier will develop interactive Graphic User Interface for creating a core shuffling (with manual, semi-automatic, automatic optimization) and Tool for generating the NFCH, Core Design Project and NFCH for physical and energetical start-up test reports, if the required modules/tools are not part of the software solution delivery in milestone M1 output D1 in the table above. Requirements for these modules/tools are specified in Requirement 2.2.24,  Requirement 2.2.25 and Requirement 2.2.34.

For the final acceptance of the software by the customer:

(a) The following scope of calculations must be performed by SE's internal forces and with the technical support of the supplier for at least one unit at the EBO site and one unit at the EMO site:

- Prepare the core design project in the scope of the template in Annex 3,

- Perform the calculation of detailed neutron-physical characteristics of the reactor core within the scope of the template in Annex 4,

- Perform the calculation of neutron-physical characteristics of the reactor core for the physical and energetical start-up tests within the scope of the template in Annex 5,

- Prepare a computational code verification report within the scope of the template in Annex 6.

(b) The supplier is obliged to deliver all Test documentation including strategy and methodology, Test cases, tracking and reporting tool, monitoring of progress and statistics, and regular outputs for management.

(c) The supplier will provide final Verification and Validation Report including uncertainties statements as specified in Requirement 1.2.1 - Requirement 1.2.5. Validation must be performed for the EBO and EMO units as an integral part of the implementation process of the delivered software solution specifically tailored for the EBO and EMO units.

(d) The supplier will provide permanent production License key (if needed).

###### Methodological and application support, assisting software users in calculations for individual SE nuclear reactors

Methodological and application assistance to help users during implementation, development and testing phases to correct use of solution including calculation of NFCH, GUI and Tool for reports generation and also IT administration task.

## Operational services

The general requirements for the operation of the information systems are defined in Annex 1 (Chapter 8, SLA2- critical), from which we derive the requirements for the Supplier:

The following services will be required and provided as part of the operation:

- Flat-rate services:

  - Analysis, diagnostics, and repair of defects as a part of the incident and problem management process.

  - SW updates.

  - Compatibility with current versions of related software products.

- Beyond Flat-rate services

###### Flat-rate services

A defect/error is an irregular or repeated behavior of a specific function that prevents the use of a given solution for its intended purpose, or is contrary to the contractual, project, or other documentation for the given solution, or is not in accordance with other changes agreed between the customer and the supplier.

Analysis and diagnostics within flat-rate services also include errors that have manifested themselves in the supplied software, the cause and solution of which lie in related operating systems, middleware, and databases.

We also consider inconsistency in the application after applying a patch to the related operating system (client or server), middleware, or database server to be a defect.

For the purpose of identifying defects, the documentation in force on the date of entry into force of this Contract or processed in the future in connection with the Supplier's activities shall be determinative.

Breakdown of defects:

| Category defects | Description |
| --- | --- |
| Category A | Urgent defect - is one without the solution of which the Work is not usable in its basic functions or there is a functional failure preventing the Work from functioning. This condition may jeopardize the Customer's normal operations or cause financial or other damages. A transitional solution by internal organisational measure is not possible. It has a restrictive impact on the support of the internal process or the compliance with the legislative obligations or contractual commitments of the Customer. |
| Category B | Serious defect - is one without the solution of which the Work is degraded in its functions in such a way that this condition restricts the normal operation and operation of HA and DR solutions. Causes problems in the use and operation of the software or its components. Allows the Work to be used without affecting the consistency of the data and the results of its processing. It can be temporarily resolved by an internal organisational measure. |
| Category C | Ordinary defect - is one which by its nature does not restrict normal operation Causes a decrease in user comfort. |

Times for defect resolution/neutralization:

| Parameter | Defect A (urgent) | Defect B (severe) | Defect C (normal) |
| --- | --- | --- | --- |
| Response time | 1 working day | 3 working days | 14 calendar days |
| Resolution / neutralization time | 2 working days | 7 calendar days | 2 month |

The duration is calculated from the reporting of the defect until the delivery of the repair. Working days are: Monday, Tuesday, Wednesday, Thursday, Friday.

If an alternative solution is provided, the priority of the defect is adequately reduced (from A to B, or from B to C).

Validation of the software must be performed by the software vendor to the extent appropriate for the defect being addressed. The modifications must be documented.

###### SW updates

Regular software updates and patching must be included to ensure the full functionality and safe operation of the supplied system. Fixes for published and detected cyber vulnerabilities.

###### Compatibility with current versions of related software products

Compatibility with current versions of operating systems, databases, middleware, and components used by third parties, which we regularly update and patch to ensure reliable and secure operation. Elimination of errors in application inconsistencies after the aforementioned updates and fixes.

###### Beyond Flat-rate services

The Bidder shall submit a price list of unit prices per man-hour by role under which sub-orders would be executed for on-call work.

The following services are the subject of the on-demand work:

- Application support beyond flat-rate services,

- Methodological support beyond flat-rate services,

- Assistance, e.g. in the creation of new calculation scenarios, in case of any ambiguities regarding calculations, etc.,

- Additional training,

- Other, by mutual agreement.

These will be specified in an individual partial Order issued by the Customer under this Contract, which must be signed by authorized representatives of both contracting parties. More detailed specifications will be determined by the Customer in the Order.

# Limits of performance, exclusion from performance and counter-performance

## Boundaries of performance

The limits of fulfilment are:

- the users' workstations on which the software is installed,

- server that communicates with workstations (if it is part of the delivery or implementation of the SW solution in the SE environment),

- Other.

The final solution depends on the architecture of the delivered software solution and cannot be specified at this stage. The same applies to the performance boundaries.

## Counterclaims

The supplier may require data to verify the correctness of the modelling.

### Documentation to be provided by SE

- Core design project (Annex 3),

- Detailed neutron-physical characteristics of the reactor core (Annex 4),

- Neutron-physical characteristics of reactor core for physical and energetical start-up tests (Annex 5),

- Computational code verification report (Annex 6).

# Annexes to the technical specification

Annex 1: ICT architectural and technological standards

Annex 2: Annex No.1 to Decree No.227/2025 Coll.

Annex 3: Core design project - content requirements

Annex 4: Detailed neutron-physical characteristics of the reactor core - content requirements

Annex 5: Neutron-physical characteristics of reactor core for physical and energetical start-up tests - content requirements

and for extended physical and energetical start-up tests - content requirements

Annex 6: Computational code verification report - content requirements